Copyright © 2020 by AL13® Architectural Systems. Review and edit according to project requirements and specifiers practice. For more information contact AL13® Architectural Systems at 855-438-2513, Info@AL13.com, www.AL13.com.

Exclusively published and distributed by AL13® Architectural Systems for editing and use by Architects and Engineers on their construction projects.

GENERAL NOTE TO SPECIFIER:

This Specification Section has been prepared to assist design professionals in the preparation of project or office master Specifications. It follows SectionFormat guidelines established by the Construction Specifications Institute (CSI), and therefore may be used with most master Specification systems with minor editing.

Edit carefully to suit project requirements. Modify as necessary and delete items that are not applicable. Verify that referenced Section titles are correct. (Titles referenced are based on CSI MasterFormat, latest edition).

This is written as a closed proprietary specification with option to allow substitutions.

Specifier/Editor Notes are in shaded boxes as hidden text and should not be printed in project issues.

Optional items requiring selection by the specifier are enclosed within brackets, e.g. [35] [40] [45]. Make appropriate selections and delete others including the brackets themselves.

Items requiring additional information are enclosed within single Guillemots, e.g. <insert option here>.

Revise header and footer to suit project/office requirements.

Electronic versions of this specification utilize automatic paragraph numbering based on Microsoft Word style formatting matching those of MasterSpec®, a product of The American Institute of Architects (AIA). To adjust levels of paragraphs assign appropriate style and numbering will automatically adjust.

TIPS:

This document includes Specifier/Editor Notes in shaded boxes as hidden text. To toggle hidden text on or off, follow these steps to display the Word Options dialog box:
In Word 2003 click Tools, Options, display the View tab, then check or uncheck Hidden Text under Formatting Marks.
In Word 2007, 2010, 2013, 2016, 2019, and Word in Microsoft 365, display the Word Options dialog box. (In Word 2007 click the Office button, then click Word Options. In Word 2010 and later versions display the File tab of the ribbon and then click Options.), then check or uncheck Hidden Text under "Always show these formatting marks on the screen".

See also https://wordribbon.tips.net/T008998_Hiding_and_Displaying_Hidden_Text.html 
and https://wordribbon.tips.net/T010510_Printing_Hidden_Text.html

Text for sustainable design requirements, such as LEED, is highlighted in green thusly. Retain or delete as required.

Revise this Section by adding, changing, and deleting text to meet Project-specific requirements.

| Release No. | Release Date | Release Title "Released for…." | Description of [Development][Changes] |
| --- | --- | --- | --- |
| 1 | 9/7/23 | Initial review | Three new sections |
| 2 |  |  |  |
| 3 |  |  |  |

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section numbers and/or titles referenced in this Section are correct for this Project's Specifications; Section numbers and titles may have changed.

SECTION 07 42 13.26 - EXTRUDED ALUMINUM PLANK WALL PANELS

- GENERAL

[Verify all notes apply
ensure reference standards are used in section]

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes extruded aluminum plank wall [and soffit] panels with supporting extrusions and clips.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Retain either Division number and Title or Section number with or without Title as standard with office practice.

          - [Division 07] Section [07 62 00] "Sheet Metal Flashing and Trim" for additional requirements.

      - REFERENCES

List abbreviations and acronyms used in this Section that are specific to the work results specified. General or typical abbreviations or acronyms may be specified in Div. 01.

        - Abbreviations and Acronyms:

If retaining optional Reference Standard below, edit to include only those references cited in the edited section.

        - Reference Standards: Perform Work according to standards specified and as follows unless modified by requirements in the Contract Documents.

Include one or both of the following options only when cost of acquiring specified standards is justified. Retain first option for paid or free subscription access. Retain second option when a physical copy on site is required.

          - [Make available via internet access] [Maintain on site] a copy of each standard affecting the Work of this Section.

The following paragraph assumes that the edition dates of reference standards will be the latest edition as of a selected date, rather than a fixed edition date. Edit to suit project conditions. If a fixed edition date is desired, delete the following and add edition dates in individual sections.

          - Publication Dates: Comply with standards [in effect as of date of the Contract Documents], unless otherwise indicated.

          - American Architectural Manufacturers Association (AAMA):

            - AAMA 508: Voluntary Test Method and Specification for Pressure Equalized Rain Screen Wall Cladding Systems.

            - AAMA 609 & 610 - Cleaning and Maintenance Guide for Architecturally Finished Aluminum (Combined Document).

            - AAMA CW-10 - Care and Handling of Architectural Aluminum From Shop to Site.

          - ASTM International (ASTM):

            - ASTM A 153/A153M: Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware.

            - ASTM A653/A653M: Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

            - ASTM B117: Standard Practice for Operating Salt Spray (Fog) Apparatus.

            - ASTM B209/B209M - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate.

            - ASTM B633: Standard Specification for Electrodeposited Coatings of Zinc on Iron and Steel.

            - ASTM B695: Standard Specification for Coatings of Zinc Mechanically Deposited on Iron and Steel.

            - ASTM C754: Standard Specification for Installation of Steel Framing Members to Receive Screw-Attached Gypsum Panel Products.

            - ASTM C920: Standard Specification for Elastomeric Joint Sealants.

            - ASTM C954: Standard Specification for Steel Drill Screws for the Application of Gypsum Panel Products or Metal Plaster Bases to Steel Studs from 0.033 in. (0.84 mm) to 0.112 in. (2.84 mm) in Thickness.

            - ASTM D2244: Standard Practice for Calculation of Color Tolerances and Color Differences from Instrumentally Measured Color Coordinates.

            - ASTM D4214: Standard Test Methods for Evaluating the Degree of Chalking of Exterior Paint Films.

            - ASTM E84: Test Method for Surface Burning Characteristics of Building Materials.

            - ASTM E136: Standard Test Method for Assessing Combustibility of Materials Using a Vertical Tube Furnace at 750 °C

            - ASTM F2329: Standard Specification for Zinc Coating, Hot-Dip, Requirements for Application to Carbon and Alloy Steel Bolts, Screws, Washers, Nuts, and Special Threaded Fasteners.

            - ASTM G87: Standard Practice for Conducting Moist SO2 Tests.

          - Deutsches Institut für Normung (DIN):

            - DIN 50018: Sulfur Dioxide Corrosion Testing In A Saturated Atmosphere.

          - National Fire Protection Association (NFPA):

            - NFPA 285: Standard Test Method for Evaluation of Fire Propagation Characteristics of Exterior Non-Load-Bearing Wall Assemblies Containing Combustible Components.

          - U.S. Green Building Council (USGBC) - Leadership in Energy and Environmental Design (LEED) (http://www.usgbc.org/):

            - USGBC-LEED [v4] [v4.1] for [Building Design and Construction (BD+C)] [Residential Design and Construction] Reference Guide.

      - ADMINISTRATIVE REQUIREMENTS

Retain "Preinstallation Meetings" Paragraph below if Work of this Section is extensive or complex enough to justify a meeting.

        - Preinstallation Meetings: Conduct at [Project site] <Insert location>.

If needed, insert list of meeting participants not mentioned in Section 01 31 00 "Project Management and Coordination."

Retain subparagraphs below if additional requirements are necessary; revise to include more specific information about conference.

          - Attendees: Contractor and its superintendent, plank system installer, representatives of plank manufacturer, and other parties involved in or affected by the installation and its coordination or integration with other materials and installations that have preceded or will follow. Advise Architect, Owner, Owner's insurer if applicable of scheduled meeting dates.

          - Review and finalize construction schedule and verify availability of materials, Installer's personnel, equipment, and facilities needed to make progress and avoid delays.

          - Inspect and discuss condition of substrate and support conditions including alignment, and other preparatory work performed by other trades.

          - Review methods and procedures related to plank installation, including manufacturer's written instructions.

          - Review structural load limitations.

          - Review flashings, special details, penetrations, openings, and condition of other construction that affect planks.

          - Review required inspecting, testing, and certifying procedures.

          - Review weather and forecasted weather conditions and procedures for coping with unfavorable conditions.

          - Review preparation, sequence of operations, and other requirements for installing work of this Section.

          - Discuss and agree upon quality control procedures.

          - Review coordination required with other Sections.

          - Review protection and repair requirements for completed work.

          - Record significant proceedings and decisions in minutes including agreements and disagreements, and required corrective measures and actions. Distribute meeting minutes to parties in attendance and others affected by proceedings and decisions made.

      - ACTION SUBMITTALS

Do not ask for manufacturer's specifications, material safety data sheets, installation instructions, and similar items that may conflict with specification requirements or needlessly involve Architect in Contractor's means and methods of construction.

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for plank wall [and soffit] system.

Include Shop Drawings only for work that involves custom fabrication or if Shop Drawings are typically prepared to assist in the performance of the work involved.

        - Shop Drawings:

          - Indicate layout, profiles and product components including dimensions, wall openings, and finish colors and textures.

          - Include details showing thickness and dimensions of flashing, the various system parts, fastening, accessories, anchoring methods, locations of joints and gaskets, and location and configuration of movement joints.

          - Include details of flashing, trim and anchorage, at a scale of not less than 1-1/2 inches in 12 inches (1:10).

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples: For each exposed product and for each color and texture specified, full plank width by [4 inches (102 mm)] <Insert dimensions> in length. Include plank manufacturer’s color charts showing full range of standard colors and finishes.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection: For each type of exposed finish required.

          - Size: [Manufacturer's standard.] [4 by 4 inches (102 x 102 mm)] [6 by 6 inches (152 x 152 mm)] <Insert dimensions>

        - Samples for Verification: For each type of exposed finish required.

          - Size: [Full plank width by 12 inches long.] <Insert dimensions>

          - Include samples of each type fastener, closure, and other plank accessories.

      - INFORMATIONAL SUBMITTALS

Requirements in first paragraph below are for test reports for products on which tests are performed either by independent testing agencies or by manufacturers in their own labs. Retain first option if testing is likely to be performed in manufacturer's facilities and witnessed by a qualified testing agency; retain second option if testing is performed by the testing agency.

        - Product Test Reports: For each type of plank system, for tests performed by [manufacturer and witnessed by a qualified testing agency] [a qualified testing agency].

Retain "Field quality-control reports" Paragraph below if Contractor is responsible for field quality-control testing and inspecting.

        - Field quality-control reports.

        - Sustainable Design [LEED] Submittals:

Green Building Certification standards vary by organization. Organizations include, but are not limited to, LEED (Leadership in Energy and Environmental Design), Energy Star, BREEAM (the Building Research Establishment Environmental Assessment Method), Green Globes, Living Building Challenge, NGBA (National Green Building Standard), GreenGuard, WELL Building Standard, TX-CHPS (Texas Collaborative for High Performance Schools) and others.

Modify wording of requirements below to suit appropriate green building certification requirements. Refer to a certified professional for additional advice on proper wording.

          - <insert requirements or reference to Division 01 Sections as required>

        - Sample Warranty: For manufacturer's warranty.

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 01 40 00 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For Installer [manufacturer] [factory-authorized service representative] <Insert entity or specialist>.

Insert other kinds of informational submittals such as design data and manufacturer's field reports as necessary for the Section.

      - CLOSEOUT SUBMITTALS

Retain one of first two paragraphs below.

        - Maintenance Data: For plank system to include in maintenance manuals.

        - Executed warranties.

      - QUALITY ASSURANCE

Include in this article prerequisites, standards, limitations, and criteria that establish an overall level of quality for products and workmanship. Include only those quality-assurance provisions that affect all Work of this Section rather than those that apply only to Part 2 - Products or Part 3 - Execution.

        - Manufacturer Qualifications: Minimum [5] <insert number> years experience[ under current organizational structure].

        - Installer Qualifications: [An entity that employs installers [and supervisors] who are trained and approved by manufacturer] [An authorized representative who is trained and approved by manufacturer] with a minimum of five years experience, who has completed systems similar in material, design, and extent to that indicated for Project and with record of successful performance.

        - Manufacturer's Technical Representative Qualifications: Minimum [5] years experience with manufacturer of Products specified in this Section.

        - Mockups: Build mockups [to verify selections made under Sample submittals] [to demonstrate aesthetic effects] [to set quality standards for materials and execution] [to set quality standards for fabrication and installation] [for preconstruction testing].

          - Build mockup of typical plank system [as shown on Drawings] <Insert size>, including [external corner,] [internal corner,] [external and internal corners,] [soffits,] supports, attachments, and accessories.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 01 40 00 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - Deliver materials and components in manufacturers’ unopened containers or bundles, fully identified by name, brand, type and grade. Prevent damage during unloading, storing and installation.

        - Store, protect and handle materials and components in accordance with manufacturer’s recommendations to prevent twisting, bending, mechanical damage, contamination and deterioration.

        - Store materials off ground on clean pallets and keep clean, dry, flat, and free of dirt and other foreign matter.

        - Do not expose planks with strippable film to direct sunlight or extreme heat.

        - Handle products of this section in accordance with AAMA CW-10.

      - FIELD CONDITIONS

        - Ambient Conditions: Proceed with installation only when existing and forecasted weather conditions permit assembly of plank system to be performed according to manufacturers' written instructions and warranty requirements.

Do not install planks where surface temperatures are anticipated to exceed 180°F (82°C).

      - WARRANTY

Include special Project warranties only in those Sections where availability has been verified with contractors and manufacturers listed.

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty: Manufacturer's standard form in which manufacturer agrees to repair or replace components of plank systems that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures including rupturing, cracking, puncturing, or excessive deflection.

            - Deterioration of metals and other materials beyond normal weathering.

Verify available warranties and warranty periods for planks.

          - Warranty Period: [15] <Insert number> years from date of Substantial Completion.

        - Special Warranty on Plank Finishes: Manufacturer's standard form in which manufacturer agrees to repair finish or replace planks that show evidence of deterioration of factory-applied finishes within specified warranty period.

Usually retain "Exposed Plank Finish" Subparagraph below for fluoropolymer finish; verify availability with manufacturer.

          - Exposed Plank Finish: Deterioration includes, but is not limited to, the following:

            - Color fading more than 5 Hunter units when tested according to ASTM D2244.

            - Chalking in excess of a No. 8 rating when tested according to ASTM D4214.

            - Cracking, checking, peeling, or failure of paint to adhere to bare metal except for slight crazing or cracking as may occur on tightly roll-formed edges or brake bends at time of forming

Verify available warranties for ACM plank finishes and insert number in "Finish Warranty Period" Subparagraph below.

          - Finish Warranty Period: [15] <Insert number> years from date of Substantial Completion.

        - Special Warranty: Furnish installer's [2] year labor warranty from [date of Owner's acceptance of completed work] to cover repair of materials found to be defective as a result of installation errors.

- PRODUCTS

For an explanation of options and Contractor’s product selection procedures, see Section 01 60 00 "Product Requirements."

      - MANUFACTURERS

        - Basis-of-Design Product: Provide AL13 Aluminum Plank System by AL13® Architectural Systems, a subsidiary of Anenda Systems Inc., Tel: 855-438-2513, Info@AL13.com, www.AL13.com.

          - Substitutions [will] [will not] be considered.

        - Source Limitations: Obtain plank system materials from single source from single manufacturer.

          - Obtain secondary and incidental components from same manufacturer as plank system or manufacturer approved by plank system manufacturer.

      - SYSTEM DESCRIPTION

Retain this article to describe complete system assembly and components or function.

        - Extruded aluminum plank system consists of the following:

          - Extruded aluminum  planks coated with high performance PVDF powder-coat fluorocarbon finish with simulated woodgrain pattern.

          - Extruded aluminum plank frame and perimeter frame assembly, including back plates, top caps, inside and outside corner frames, and end frames.

          - Galvanized steel system clips.

Retain optional subparagraph below if desired.

          - Extruded aluminum snap-lock top cap with high performance fluorocarbon finish for final attachment of planks to frame assembly.

      - PERFORMANCE REQUIREMENTS

        - Design Criteria: Design extruded aluminum plank wall [and soffit] panel system using performance requirements and design criteria specified herein.

          - Determine component and cladding wind loads per ASCE/SEI 7, "Minimum Design Loads and Associated Criteria for Buildings and Other Structures."

          - Design to span continuously over at least four structural supports (three spans).

          - Design fastening to structural supports to withstand factored loads in accordance with authority having jurisdiction and the project structural design.

          - Install expansion joints to accommodate movement in wall system and between wall system and building structure, where these movements are caused by deflection of building structure, and accommodate these movements, without permanent distortion, damage to infills, racking of joints, breakage of seals, or water penetration.

          - Provide for positive drainage to the exterior of all water entering or condensation occurring within the system.

        - Structural Performance Requirements: Provide extruded aluminum plank wall [and soffit] system, including anchorages, capable of withstanding, without failure, the effects of gravity loads and the following loads and stresses within limits and under conditions indicated, determined per the following code requirements:

          - Design Wind Loads: [As indicated on Drawings.] <insert as required>

          - Deflection Normal to Wall Plane: For continuous aluminum members, total load deflection shall not exceed L/180 of clear span. Install panel stiffeners in compliance with the manufacturer’s installation instructions if required to meet deflection requirement.

Fire-test-response characteristics in first two paragraphs below are generally included to ensure compliance with regulatory requirements. Place characteristics here if they are applicable to all products in this Section and their performance, once installed, is part of a fire-rated assembly. Place characteristics in individual product descriptions if each product has different characteristics and performance, or locate overall criteria, including test methods and listing requirements, here and product-specific performance values in each product description. Never use terms such as "fire-hazard classification" or others implying some degree of fire-risk assessment.

        - Surface-Burning Characteristics: Comply with ASTM E 84; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

          - Smoke Developed: 0

          - Flame Spread: 0

        - Fire Propagation Characteristics: Plank system passes NFPA 285 testing.

          - Combustibility - ASTM E136 Option A: passed

        - Rainscreen Performance Characteristics: Per AAMA 508.

          - Pressure Equalization: Pass.

          - Static Water Penetration: Pass at 15 psf .

          - Dynamic Water Penetration: Pass.

          - Structural Loading: Pass at 105 psf design pressure.

        - Thermal Movements: Allow for thermal movements from ambient and surface temperature changes.

Differential values in "Temperature Change" Subparagraph below (for aluminum in particular) are suitable for most of the U.S.

          - Temperature Change: [120 deg F (67 deg C), ambient; 180 deg F (100 deg C), material surfaces] <Insert temperature change>.

      - EXTRUDED ALUMINUM PLANK WALL [AND SOFFIT] PANELS

        - Extruded Aluminum Plank System: Factory-formed and field-assembled, planks fabricated from two metal facings bonded to [polyethylene core] [fire-rated mineral core]

Custom sizes available up to 5 feet x 10 feet (1524 mm x 3048 mm).

          - Aluminum Plank Material: ASTM B209, ANSI H35.1 alloy designation 6063 T5 tension leveled, aluminum

          - Plank Width: [4 inches (102 mm)] [6 inches (152 mm)] [2 inches (51 mm)].

          - Plank Overall Thickness: 9/16 inch (14 mm).

[Please verify the following thickness of the aluminum sheet used - not the whole plank]

          - Minimum Plank Metal Thickness: [0.040 inch (3 mm)] [0.063 inch (4 mm)].

Retain first value option below for 4 inch planks. Retain second value option below for 6 inch planks. Retain third value option below for 2 inch planks

          - Plank Weight: [0.50 lb/sq ft (2.44 kg/sq m)] [0.69 lb/sq ft (3.67 kg/sq m)] [0.234 lb/sq ft (1.14 kg/sq m)]

          - Profile: [Smooth] [Wood grain].

          - Exterior Finish: Two-coat fluoropolymer with simulated woodgrain pattern.

            - Color: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of [manufacturer’s standard] [available industry] colors].

        - Trim Components: Formed from extruded aluminum.

      - ATTACHMENT FRAMING

        - Girts and Subgirts: ASTM A653, Grade 230 cold-formed, metallic-coated steel sheet, minimum 0.0516 inch (1.27 mm) thick, with ASTM A653/A653M, G90 (Z275 hot-dip galvanized) coating designation, finished to match planks where visible in finished work.

          - Subgirts: Adjustable double-angle profile as indicated to accept plank with structural attachment to building frame.

      - ACCESSORIES

        - Plank Accessories: Provide components required for a complete, rainscreen plank system including trim, copings, fasciae, mullions, sills, corner units, clips, flashings, sealants, and similar items. Match material and finish of planks unless otherwise indicated.

        - Fasteners for System Clips: Corrosion-resistant-coated, self-drilling screws designed to withstand design loads of size of type as recommended in writing by plank manufacturer. Provide EPDM or PVC sealing washers for exposed fasteners with heads matching color of Planks by means of plastic caps or factory-applied coating.

          - Screws for attachment to steel framing: ASTM C954; corrosion resistant, hex-head, self-drilling self-tapping type with EPDM composite washers, minimum No. 12 diameter by length required to penetrate steel with not less than three exposed threads, zinc-coated steel as specified below.

            - Acceptable Product: AL13 Hex-Head Fastener, #12-14 x 1-1/2.

          - Screws for attachment to wood substrates: ASTM C954; corrosion resistant, hex-head, self-drilling type with EPDM composite washers, minimum No. 12 diameter by length required to penetrate not less than 1-1/4 inch (32 mm) into wood, coated steel as specified below.

            - Acceptable Product: AL13 Hex-Head Fastener, #12-14 x 1-3/4.

[Verify if below is required]

For large areas installed over concrete, installing a furring bar or Z-girt for plank attachment is recommended due to time consuming nature of attaching frame components directly to concrete.

          - Screws for attachment to concrete and masonry: Alternating high-low thread form type, hex-head, self-cutting threads with matched tolerance drill bit, minimum 1/4 inch diameter by length required to penetrate not less than 1-1/4 inch (32 mm) into concrete, stainless steel as specified below.

            - Acceptable Product: AL13 Hex-Head Fastener, 1/4 x 1-1/2 inch.

        - Fasteners for Frame Components: Corrosion-resistant-coated, self-drilling screws designed to withstand design loads of size of type as recommended in writing by plank manufacturer. Provide EPDM or PVC sealing washers for exposed fasteners with heads matching color of Planks by means of plastic caps or factory-applied coating.

          - Screws for attachment to steel framing: ASTM C954; corrosion resistant, hex-head, self-drilling self-tapping type, minimum No. 10 diameter by length required to penetrate steel with not less than three exposed threads, zinc-coated steel as specified below.

            - Acceptable Product: AL13 Hex-Head Fastener, #10-16 x 3/4.

          - Screws for attachment to wood substrates: ASTM C954; corrosion resistant, hex-head, self-drilling type with EPDM composite washers, minimum No. 12 diameter by length required to penetrate not less than 1-1/4 inch (32 mm) into wood, coated steel as specified below.

            - Acceptable Product: AL13 Hex-Head Fastener, #12-14 x 1-1/2.

[Verify if below is required]

For large areas installed over concrete, installing a furring bar or Z-girt for plank attachment is recommended due to time consuming nature of attaching frame components directly to concrete.

          - Screws for attachment to concrete and masonry: Alternating high-low thread form type, hex-head, self-cutting threads with matched tolerance drill bit, minimum 1/4 inch diameter by length required to penetrate not less than 1-1/4 inch (32 mm) into concrete, stainless steel as specified below.

            - Acceptable Product: AL13 Hex-Head Fastener, 1/4 x 1-1/2 inch.

          - Screws for insulation fastening: Type appropriate to substrate above with 1-1/2-inch (38 mm) diameter washer of length required to penetrate minimum one inch (25 mm) into framing or substrate with 1-1/2-inch (38 mm) diameter washer, zinc-coated steel as specified below.

          - Material by Location:

            - Screws in contact with concrete and masonry: 300 Series (304, 305, or 316) stainless steel.

            - Screws in contact with [preservative-treated wood] [or] [fire-retardant-treated wood] in exterior applications or wet or damp locations: ASTM A 153 or ASTM F2329 hot-dipped zinc-coated galvanized steel or stainless steel.

            - Other Screws: Zinc-plated carbon steel per ASTM B695 or ASTM B633 with polymer-coating able to withstand salt-spray resistance to red rust of more than 1,700 hours per ASTM B117, 18 cycles of ASTM G87 (DIN 50018) SO2 Kesternich testing with not more than 15 percent red rust or stainless steel.

Retain optional "Adhesive Tape" paragraph below if desired for added plank rigidity.

        - [Adhesive Tape: As recommended in writing by plank manufacturer for permanent adhesion of I-beam stiffeners to backs of Planks.]

          - Acceptable Product: AL13 Adhesive Tape by Anenda Systems Inc.

        - Isolation Tape: Manufacturers standard material for separating dissimilar metals from direct contact.

Retain either Division number and Title or Section number with or without Title as standard with office practice.

        - Insulation: Rigid type as specified in [Division 07] Section [07 21 13] "Thermal insulation.

        - Flashing and Trim: Provide flashing and trim formed from same material as planks, minimum 0.062 inch (1.57 mm) thickness and per SMACNA recommendations as required to seal against weather and to provide finished appearance. Where exposed to view, finish to match adjacent planks. Locations include, but are not limited to, bases, drips, sills, jambs, corners, endwalls, framed openings, rakes, fasciae, parapet caps, soffits, reveals, and fillers. Finish flashing and trim with same finish system as adjacent planks.

Retain either Division number and Title or Section number with or without Title as standard with office practice.

        - Air/Vapor Retarder: As specified in [Division 07] Section [07 27 13] "Air Barriers."

        - Gaskets: Medium durometer Santoprene or EPDM.

Retain either Division number and Title or Section number with or without Title as standard with office practice.

        - Sealants: ASTM C920; elastomeric polyurethane or silicone sealant compatible with adjacent materials; of type, grade, class, and use classifications required to seal joints in planks and remain weathertight; complying with requirements in [Division 07] Section [07 90 00] "Joint Sealants," and as recommended in writing by plank manufacturer.

          - Color: [Match planks] [As selected by Architect from full range of available industry custom or standard colors].

        - Bituminous Paint: Cold-applied asphalt-mastic paint complying with SSPC-Paint 12 requirements except containing no asbestos, formulated for 30-mil thickness per coat.

      - FABRICATION

        - General: Extrude and finish planks and accessories to indicated profiles at the factory, by manufacturer's standard procedures and processes. Field cut planks within dimensional tolerances.

        - Field Measurements: Verify location of structural members and openings in substrates by field measurements before fabrication and indicate measurements on Shop Drawings. Coordinate fabrication schedule with construction progress to avoid delaying the Work

        - Flashing and Trim: Form metal flashing and trim to comply with plank manufacturer's recommendations and recommendations in SMACNA's "Architectural Sheet Metal Manual" allowing for thermal expansion.

Revise SMACNA Figure below if another seam type is desired.

          - Provide butt seams with backup plates, fastened one side and sealed with butyl sealant (SMACNA Fig. 3-3, Style 19).

          - Conceal fasteners and expansion provisions where possible.

        - Fabrication Tolerances:

          - Plank bow shall not exceed 0.8 percent of plank overall dimension in width or length.

          - Plank dimensions shall allow for field adjustment and thermal movement.

          - Plank lines, breaks and curves shall be sharp, smooth and free of warps or buckles.

          - Plank shall be visually flat.

          - Plank surfaces shall be free of scratches or marks caused during fabrication or installation.

      - FINISHES

        - High-Performance Organic PVDF Extrusion Finish: 2-coat polyester or polyurethane powder-coated color finish with simulated woodgrain pattern heat-transfered by sublimation process. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating manufacturers' written instructions.

Available woodgrain colors include Douglas fir, maple, oak, light European cherry, cherry, and dark natural walnut.

          - Basis of Design Product: Decoral Process by Decoral System USA Corp.

          - Color: [As [indicated] [ or, if not indicated, as ]selected from manufacturer's full range.] [Match Architect's sample.]

        - Protect finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

- EXECUTION

      - EXAMINATION

        - Verification of Conditions:

          - Examine substrates, areas, and conditions[, with installer present,] for compliance with requirements for installation tolerances, operational clearances, and other conditions affecting performance of the Work.

          - Examine wall sheathing to verify that sheathing joints are supported by framing or blocking and that installation is within flatness tolerances required by plank manufacturer.

Retain subparagraph below with subparagraph above for systems that depend on air- or water-resistive barriers to prevent air infiltration or water penetration.

            - Verify that air- or water-resistive barriers have been installed over sheathing or backing substrate to prevent air infiltration or water penetration.

          - Examine roughing-in for components and assemblies penetrating planks to verify actual locations of penetrations relative to joint and seam locations of planks.

          - Examine planks before installation. Reject components that are marred, damaged, or deteriorated beyond ability to be touched up as specified below.

          - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Miscellaneous Supports: Install girt [and subgirt] framing, and other miscellaneous plank support members and anchorages according to ASTM C754 and plank manufacturer's written recommendations.

        - Where aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous paint.

      - INSTALLATION

        - Comply with plank manufacturer's written installation instructions and recommendations [and approved submittals].

          - Drill 0.25-inch (6.35 mm) drainage weep holes length of horizontally oriented bottom starter trim located at base of wall areas at not more than [24 inch (610 mm)] [36 inch (914 mm)] spacing.

          - Adjust assembly to secure planks safely to wall while allowing for expansion and contraction of components.

        - Do not cut, trim, weld, or braze component parts during installation in manner which would damage finish, decrease strength, or result in visual imperfection or failure in performance. Return component parts which require alteration to shop for further fabrication, if possible, or for replacement with new parts.

[Is below meant to be on top of the planks or underneath to conceal the backup wall? A detail would be helpful.]

        - At control and expansion joints, cover visible backing substrates [0.032 inch (0.81 mm)] [0.040 inch (1.02 mm)] <insert thickness> thick sheet metal matching adjacent planks in width to conceal backup structure. Fasten to only one side of joint to allow joint movement.

        - Seal junctions with adjoining work with approved sealant in accordance with [Division 07] Section [07 90 00] "Joint Sealants."

        - Erection Tolerances: Comply with the following non-accumulating maximum tolerances:

          - Level: Variation from plane or location shown on shop drawings: 0.4 inches in 33 feet (10 mm in 10 m) length to maximum of 0.79 inches in 328 feet (20 mm over 100 m).

          - Plumb: Deviation of vertical and horizontal members: 0.12 inches in 28-feet (3 mm maximum in 8.5 m) run.

          - Alignment:

            - Where surfaces abut in line or are separated by reveal or protruding element up to 1/2 inch (12.5 mm) wide, limit offset from true alignment to 1/16 inch (1.6 mm).

            - Where surfaces are separated by reveal or protruding element from 1/2 to 1 inch (12.5 to 25 mm) wide, limit offset from true alignment to 1/8 inch (3.2 mm).

            - Where surfaces are separated by reveal or protruding element of 1 inch (25 mm) wide or more, limit offset from true alignment to 1/4 inch (6.4 mm).

          - Location: Limit variation from plane to 1/4 inch in 20 feet (6.4 mm in 6 m); 1/2 inch (12.5 mm) over total length.

      - REPAIR

Div. 01 Section "EXECUTION" covers general requirements for repairing, replacing, or restoring defective and damaged new and existing work other than operating products, systems and equipment.

Describe how existing work is to be repaired and/or restored.

        - Repairing Damaged Finishes: Immediately after installation, repair areas where coatings or finishes are marred, abraded, deteriorated, or otherwise damaged.

          - Factory-Finished Surfaces: Clean abraded areas and repair with same material used for factory finish per manufacturer's written instructions.

            - Use only materials and procedures recommended or furnished by plank manufacturer.

            - Touch Up: Repair marred, damaged, or deteriorated surfaces to blend inconspicuously with adjacent unrepaired surface so touchup is not visible from a distance of 48 inches (1219 mm) as viewed by Architect. Apply with an artist brush or by spray to provide a minimum 2.0-mil (0.058 mm) dry film thickness.

            - Replace components that cannot be restored to factory-finished appearance [or that have been damaged or have deteriorated beyond successful repair by finish touchup or similar minor repair procedures] and that cannot be repaired to Architect's and Owner's satisfaction.

      - FIELD QUALITY CONTROL

Retain "Manufacturer's Field Service" Paragraph below to require a factory-authorized service representative to perform tests and inspections.

        - Manufacturer's Field Service: Engage a factory-authorized representative to observe and inspect components and assembly installations and provide recommendations.

          - Require product manufacturers to provide periodic field surveillance of the handling, installation/application, protection and cleaning of their products.

Delete below if retaining full-time surveillance above.

          - Schedule periodic site visits to review work at the following times:

            - After delivery and storage of products and after preparatory work on which work of this Section depends is complete, but before installation begins.

            - [Twice] <insert as required>during progress of work at [25] <insert as required> and [60] <insert as required> percent complete.

            - Upon completion of work, after cleaning is carried out to provide final review and acceptance of work of this Section accompanied by the [Architect] [and] [Owner].

          - Monitor and report installation procedures, weather and temperature conditions, and unacceptable conditions.

          - Prepare inspection reports.

      - CLEANING

        - Remove protective film from planks within 60 days after installation is complete.

        - Ensure weep holes and drainage channels are unobstructed and free of dirt and sealants.

        - Aluminum Surfaces: Upon completion of installation, thoroughly clean aluminum surfaces in accordance with AAMA 609 & 610.

END OF SECTION 07 42 13.23